| SEMAE3094 Carrying out maintenance of aircrew protective helmets and electrical headsets |  |
| --- | --- |

Overview

Performance criteria

You must be able to:

- work safely at all times, complying with health and safety and other relevant regulations, directives and guidelines

- follow the relevant maintenance schedules to carry out the required work

- carry out the maintenance activities within the limits of your personal authority

- carry out the maintenance activities, and replace components in the specified sequence and in an agreed timescale

- report any instances where the maintenance activities cannot be fully met or where there are identified defects outside the planned schedule

- dispose of waste materials in accordance with safe working practices and approved procedures

- complete the relevant documentation, in accordance with organisational requirements

- leave the work area, and equipment in a safe and appropriate condition, free from foreign object debris on completion of the activities

Knowledge and understanding

You need to know and understand:

Scope/range related to performance criteria

​

- Carry out all of the following during the servicing and maintenance of the aircrew protective helmets and electrical headsets:

  - obtain and use the appropriate documentation (such as job instructions, servicing or maintenance schedule, specifications, material data sheets and other relevant documentation)

  - adhere to procedures or systems in place for risk assessment, COSHH, personal protective equipment and other relevant safety regulations and procedures to realise a safe system of work

  - provide and maintain a safe working environment for the maintenance activities

  - obtain the correct tools and equipment for the activity and check that they are in a safe, tested and usable condition and within current certification/calibration date

  - obtain clearance to work on the aircraft and observe any power isolation procedures

  - use approved servicing and maintenance techniques at all times

  - return all tools and equipment to the correct location on completion of the activities

- Carry out all of the following repair/maintenance activities, using appropriate methods and techniques:

  - dismantling equipment to component or sub-assembly level (such as removal of oxygen mask, visor cover and fabric covers)

  - cleaning the equipment (such as visor, protective shell and headsets), using appropriate solutions

  - monitoring the condition/deterioration of components

  - replacing all damaged or defective components

  - reassembling the equipment

  - carrying out any required modifications to the equipment

  - carrying out adjustments to components and connections (such as friction settings, tuning and adjusting microphones)

  - checking equipment operation and performance

  - testing equipment in accordance with the relevant specification

- Carry out a thorough examination of the helmet and headset, to include checking all of the following:

  - the protective shell for damage and softness of shell

  - all protective shell screws/fasteners for security

  - visors for scratches, abrasions and cracks

  - visor hinge mechanism for corrosion, damage, security, and adjust friction settings, as required

  - ear capsules for damage, wear, hardening, discoloration and security

  - 'Mic/Tel' leads for deterioration or fraying

  - chin and neck strap for wear, damage, fraying and deterioration

  - oxygen mask hooks for damage, security and bending/distortion

  - electrical headsets for signs of damage and deterioration

  - ear pads and headbands for hardening or cracking

  - the ear shells move freely in their stirrups

  - the microphone switch moves freely, and adjustable parts move freely without undue slackness

  - boom microphone (if fitted) for looseness and damage

- Replace components, to include six of the following:

  - visor (clear or tinted)

  - pads (crown, brow or neck)

  - side arm (outer and inner)

  - elastic straps

  - base assembly oxygen mask hook

  - ear capsule tensioning webbing

  - strap assembly cable retaining

  - transducer

  - strap assembly (chin or neck)

  - microphone switch

  - headset electrical

  - microphone boom

  - ear capsule

  - earphone

  - down lead assembly

  - fabric cover

  - lining assembly (brow or neck)

  - headband

  - visor cover assembly

  - earphone shell

  - 'Mic/Tel' lead (down lead and jack plug connector)

- Carry out maintenance requirements, in accordance with two of the following types of instruction:

  - Urgent Technical Instructions (UTI)

  - Routine Technical Instructions (RTI)

  - Maintenance Instructions (MI)

  - Preliminary Warning Instructions (PWI)

  - Serious Defect Signals

- Carry out maintenance work in compliance with one of the following standards:

  - Civil Aviation Authority (CAA)/European Aviation Safety Agency (EASA)

  - Ministry of Defence (MoD)

  - Military Aviation Authority (MAA)

  - Aerospace Quality Management Standards (AS)

  - Federal Aviation Authority (FAA)

  - BS, ISO or BSEN procedures

  - customer standards and requirements

  - company standards and procedures

  - manufacturers standards and procedures

- Complete the relevant maintenance records, to include one from the following and pass it to the appropriate people:

  - maintenance schedule/log

  - job cards

  - aircraft service/flight log

  - other specific recording method

Behaviours

​

Developed by

Enginuity

Version Number

3

Date Approved

30 Mar 2021

Indicative Review Date

01 Mar 2024

Validity

Current

Status

Original

Originating Organisation

Enginuity

Original URN

SEMAE3094

Relevant Occupations

Engineer, Engineering, Engineering and Manufacturing Technologies, Engineering Technicians

Suite

Aeronautical Engineering Suite 3

Keywords

engineering; aeronautical; maintenance; aircrew protective helmet; electrical; headset; cleaning; dismantle; replace components