| SEMME3200 Carrying out manual splicing operations |  |
| --- | --- |

Overview

This standard identifies the competences you need to produce splices in wire and fibre ropes manually, in accordance with approved procedures.  You will be required to produce a range of different splices in wire and fibre ropes, in accordance with specifications or instructions.

You will be expected to obtain any necessary equipment and to check that it is in a safe and usable condition. You will need to set the rope up in a suitable workholding device (where appropriate) and prepare the rope by cutting it correctly, un-laying the strands and applying whippings. You will also be expected to a produce a tight splice, with the relevant number of tucks and locking tucks, consistent with the type of splice or the intended use.

Your responsibilities will require you to comply with organisational policy and procedures for the splicing activities undertaken, and to report any problems with these activities, or with the ropes or equipment used that you cannot personally resolve, to the relevant authority. You will be expected to work with a minimum of supervision, taking full responsibility for your own actions and for the quality of the work that you carry out.

Your underpinning knowledge will provide a good understanding of your work and will provide an informed approach to applying manual splicing procedures.  You will understand the techniques and methods of splicing and their application and you will have an adequate understanding of the properties of wire and fibre ropes, to provide a sound basis for carrying out the splicing activity, correcting faults and ensuring that the completed splice is to the required specification.

You will understand the safety precautions to be taken when carrying out the splicing activities. You will be required to demonstrate safe working practices throughout and will understand the responsibility you owe to yourself and others in the work place.

Note

:  All spiced ropes will have to be tested by the appropriate authorities, in accordance with their operating environment and intended use.

Performance criteria

You must be able to:

- work safely at all times, complying with health and safety and other relevant regulations, directives and guidelines

- obtain the required equipment and materials and check that they are in a safe and usable condition

- follow instructions and any relevant specifications to produce the splice

- produce the required splice using appropriate methods and techniques

- check that the finished splice meets the requirements and make any necessary adjustments

- deal promptly and effectively with problems within your control and report those that cannot be solved

- ensure that all the ropes that have been spiced are tested by the appropriate authorities before use

- complete the required recording and register

- documentation in line with organisational

- procedures,

- after the splicing operation

Knowledge and understanding

You need to know and understand:

- the specific safety precautions to be taken when carrying out splicing operations with regard to health and safety legislation, personal protective equipment (PPE), safe working practices and procedures and the safety of yourself and others working in the vicinity

- the risks and hazards associated with carrying out splicing operations, and how they can be minimised

- the correct handling and storage of wire and fibre ropes

- drawings, specifications and work instructions relating to splicing

- types of splice, their purpose and intended use (to include short splice, long splice, back splice and eye splices)

- tests to be applied to particular splices

- the limitations and constraints associated with the respective types of splice

- breaking strain and safe working load (SWL) or working load limit (WLL) of wire and fibre ropes and how this is affected by splicing

- the tools, equipment and consumables used in the splicing operations

- cutting and unlaying wire and fibre rope

- the type and purpose of whippings in the splicing operation

- workholding devices used when splicing

- the sequence, number and type of tucks to be made

- completing a splice by whipping or worming, parcelling and serving; its purpose and under what circumstances they would be used

- checking all aspects of a completed splice

- the requirements regarding testing the completed spliced ropes and the appropriate authority to carry out these tests (taking into account to the environment in which they are to be used and the use/load they are intended to lift/move)

- completing records and registers of rigging equipment

- the problems that can occur with splicing operations and how they can be avoided

- the extent of your own responsibility and whom you should report to if you have problems that you cannot resolve

Scope/range related to performance criteria

​

1.    Prepare for the splicing activities, to include carrying out

all

of the following:

- obtain the specification for the splice (including type of splice, size of eye (if applicable), safe working load (SWL) or working load limit (WLL) required, number of tucks)

  - obtain and check the necessary equipment (including workholding device, if applicable)

  - obtain the wire or fibre rope, as specified

  - cut the wire or fibre rope to the required length

2.    Prepare the wire and fibre rope for splicing, to include carrying out

all

of the following:

- whip the rope at the extent to which it is to be unlaid

  - unlay and whip the ends of unlaid strands

  - whip the rope in way of the first tuck (wire rope, eye splice)

  - secure wire rope in the workholding device (where applicable)

  - for eye splice (where applicable), whip the thimble in place

3.    Carry out

two

of the following splices in wire ropes:

- eye splice

  - short splice

  - long splice

  - other specific splice

4.    Carry out

three

of the following splices in fibre rope:

- eye splice in multi-plait fibre rope

  - short or butt splice in multi-plait fibre rope

  - eye splice in fibre rope

  - back splice in fibre rope

  - short splice in fibre rope

  - long splice in fibre rope

  - other specific splice

5.    Check the completed splice against specification, for

all

of the following:

- number and type of tucks

  - sequence in which strands have been spliced

  - tightness of the splice

  - deformation and kinks

  - dimensions (including size of eye)

  - thimbles are secure in eye (where applicable)

  - SWL/WLL is adequate for the intended purpose

6.    Complete a splice by

one

of the following methods:

- whipping

  - worming, parcelling and serving

Behaviours

​

Behaviours:

You will be able to apply the appropriate behaviours required in the workplace to meet the job profile and overall company objectives, such as:

- strong work ethic

- positive attitude

- team player

- dependability

- responsibility

- honesty

- integrity

- motivation

- commitment

Developed by

Enginuity

Version Number

3

Date Approved

31 Mar 2019

Indicative Review Date

29 Apr 2021

Validity

Current

Status

Original

Originating Organisation

Semta

Original URN

SEMME3200

Relevant Occupations

Marine Engineering Trades

Suite

Marine Engineering Suite 3

Keywords

Engineering; marine; manual; splicing; rope; wire; fibre; eye; short