| IMILT01 Carry out routine lift truck maintenance |  |
| --- | --- |

Overview

This standard is about conducting routine maintenance, adjustment and replacement activities as part of the periodic servicing of fork lift trucks.

Performance criteria

You must be able to:

P1

‍

use suitable personal protective equipment throughout all lift truck maintenance activities

P2

‍

use suitable

sources of technical information

to support all your lift truck maintenance activities

P3

‍

use the correct specifications and tolerances appropriate to the lift truck you are working on, when making

assessments

of system and component performance

P4

‍

record details accurately and use this adapted specification as the basis for your examination and assessment where the lift truck being worked upon falls outside the manufacturer’s original specification

P5

‍

examine the lift truck’s systems and components following: