| IMIVBR13 Remove and replace motor vehicle body panels including permanently fixed panels |  |
| --- | --- |

Overview

This standard is about removing a variety of exterior and sub-structure body panels and panel sections, including permanently fixed panels, where these are damaged and replaced with new or repaired replacements.

The ability to join vehicle panels by welding, bonding and mechanical fastening (alone and combined) is required.

Performance criteria

You must be able to:

P1

‍

identify component materials involved in the construction of the vehicle in the areas that will be worked on during repair, prior to working on the vehicle

P2

‍

wear suitable personal protective equipment (PPE) throughout all vehicle body panel removal and replacement activities

P3

‍

inspect, prepare and use all the

tools and equipment

required, following manufacturers' instructions, prior to use

P4

‍

remove, replace and/or refit all necessary vehicle

body panels

and assemblies following:

P4.1

‍

the manufacturer's

methods

/instructions

P4.2

‍

recognised researched repair

methods

P4.3

‍

your workplace procedures

P4.4

‍

health, safety and legal requirements

P5

‍

promptly seek guidance from the relevant person(s) where there is the potential for your work to disturb other vehicle systems

P6

‍

use replacement

body panels

and assemblies which conform to the vehicle specifications for dimensions, materials and functional capability

P7

‍

use and apply sealants and weld primers and anti-corrosion treatments conforming to the material or vehicle manufacturer's specification

P8

‍

ensure all test weld pieces conform to the current British Standard for appearance and penetration

P9

‍

ensure permanently fixed panels are replaced without incurring damage to the vehicle systems

P10

‍

ensure all refitted

body panels

are aligned correctly with adjacent panels and fittings to manufacturers tolerances (panel gaps)

P11

‍

complete all removal and replacement activities within the agreed timescale

P12

‍

promptly report any anticipated delays in completion to the relevant person(s)

Knowledge and understanding

You need to know and understand:

K1

‍

the health and safety legislation and workplace procedures relevant to workshop practices, personal and vehicle protection when removing and replacing vehicle

body panels

K2

‍

the dangers of cross contamination of material such as aluminium and steel

K3

‍

the requirements of manufacturer's warranty agreements

K4

‍

the vehicle work specification agreed

K5

‍

your workplace procedures for the referral of problems, reporting of delays to the completion of work and personal protection

K6

‍

the requirements for protecting the vehicle and contents from damage before, during and after removing and replacing vehicle

body panels

*

K7

‍

the importance of working to agreed timescales and keeping others informed of progress

K8

‍

the relationship between time, cost and profitability

K9

‍

the importance of promptly reporting anticipated delays to the relevant person(s)

K10

‍

how to prepare, test and use the *

tools and equipment

required for the removal and replacement of vehicle

body panels

and ancillary fittings

K11

‍

how to operate resistance spot welding and metal inert gas (MIG), metal active gas (MAG) welding equipment to achieve welds to the current British Standard

K12

‍

how to test Resistance Spot weld strength

K13

‍

how to carry out bonding/riveting cold repairs

K14

‍

the properties of component materials involved in the construction of the vehicle in the areas that will be worked on during repair

K15

‍

the properties of sealants, adhesives and anti-corrosion materials and the requirements for their safe use

K16

‍

the type of sealants and anti-corrosion materials to use and the manufacturer's recommended methods for their application and thickness

K17

‍

how to use adhesive bonding materials

K18

‍

how to select and apply sealants and anti-corrosion materials

K19

‍

the principles of chassis frame and monocoque vehicle construction

K20

‍

how to remove vehicle manufacturers original joining techniques

K21

‍

how to identify manufacturer's joining techniques and how they may differ to the repair method

K22

‍

the principles of thermal and non-thermal joining techniques ie spot welding, metal inert gas (MIG), metal active gas (MAG), bonding, hybrid joining etc

K23

‍

the different types of mechanical fixings for vehicle

body panels

and when and why they should be used

K24

‍

the repair and welding implications of working with galvanised coatings, mild steels, HSS, UHSS and aluminium alloys

K25

‍

how panel removal and refitting affects the overall body structure of the vehicle

K26

‍

the causes and rectification of distortion resulting from welding

K27

‍

how to find, interpret and use sources of information relevant to the removal and replacement of vehicle

body panels

and assemblies

K28

‍

how to remove and replace vehicle

body panels

and assemblies

K29

‍

how to remove and replace door skins

K30

‍

how to establish cut lines for partial panel replacement

K31

‍

how to prepare all edges to be joined

K32

‍

how to select the correct joints and joining processes to match the repair area

K33

‍

the importance and implications of panel clamping and alignment to match existing contours and gaps

K34

‍

how to work safely avoiding damage to the vehicle and its systems

K35

‍

the importance and implications of checking the accuracy of repair work

K36

‍

the types of quality control checks that can be used to ensure correct alignment and contour of panels and the operation of components to manufacturer's specification

K37

‍

the methods of storing removed components and the importance of storing them correctly and in accordance with legal requirements

Scope/range

All of the items listed below form part of this National Occupational Standard.

1.

‍

Body panels

are: