| PROPF352 Set up and control in-line insetting-stitching-trimming machines |  |
| --- | --- |

Overview

​

This standard is for print finishers using in-line insetting-stitching-trimming machinery. They will be expected to set up the equipment and control it whilst running production jobs.

This standard consists of two elements:

- Set up in-line insetting-stitching-trimming machinery

- Run in-line insetting-stitching-trimming machinery and monitor quality

This is what the standard covers:

- identifying the job requirements

- checking that the in-line insetting-stitching-trimming machinery is working properly

- checking that safety devices are working properly

- running the in-line insetting-stitching-trimming machinery safely

- adjusting settings, where necessary to maintain the required standard

- checking that work meets the required standard

- identifying faults and taking action to deal with them

- unloading and stacking the finished product

Performance criteria

You must be able to:

​

Set up in-line insetting-stitching-trimming machinery

- check that you have all the job details you need

- check that you have enough materials of the right type to meet job requirements

- report if materials provided are not correct or sufficient in accordance with organisational procedures

- set up the line in accordance with organisational requirements:

- sections are inserted to give the correct page sequence

- sections and covers are fed into the stitcher squarely and without damage

- stitched books hold firmly and are securely clenched

- check that stitched books are:

- fed into the trimmer squarely and without damage

- trimmed evenly and squarely to the correct size, without marking

- delivered without damage or distortion

- check that samples produced by the machine match the required standard

- make any necessary adjustments to enable standards to be met

- report if standards cannot be met in accordance with organisational procedures

- prepare your work area so that it is safe and ready for production

- *

- *

- **Run in-line insetting-stitching-trimming machinery and monitor quality

**

10. run in-line insetting-stitching-trimming machinery so that it is safe and efficient and at the required speed in accordance with organisational procedures

11. keep up sufficient supplies of materials so that runs continue as long as necessary to meet job requirements

- check at regular intervals that your company's quality standards are met

- identify the causes of production faults, which result in:

- stitch legs of unequal length

- stitch legs not closed

- stitches missing

- marks on book spines

- adjust machines, using approved method and equipment, to correct faults which it is your job to remedy

- use agreed procedures to report faults which it is not your job to correct

- check that machinery is safe to operate, once faults have been corrected, in accordance with organisational procedures

- record the production and quality assurance details, checking that information meets organisational procedures

- follow the correct procedures for the removal of waste

- stack work using the approved method

Knowledge and understanding

You need to know and understand:

​

- your duties and responsibilities for health and safety as defined by any specific legislation covering your job role

- regulations such as those covering manual handling, noise at work, personal protective equipment, safe handling of equipment and materials, and the safe use of computer equipment

- workplace policies and written operating procedures relating to written health and safety policy statement, provision, use and processes of workplace equipment, training, prohibited equipment, young persons, safe systems of work

- the way you actually do your job, more particularly the activities and techniques and the way that materials and equipment are used

- typical hazards and risks in the printing industry and those that relate to your own job

- risk assessment techniques and the action to take to deal with them

- codes of practice relevant to your role and where to obtain information on them

- manufacturers' and suppliers' health and safety instructions and advice for operating machinery, guarding machinery and data sheets for substances harmful to health

- the requirements for personal presentation including personal hygiene, suitable clothing and accessories, fitness for work, such as not under the influence of drugs, alcohol or medication, smoking policies in the workplace

- how to stop a machine in the event of an emergency

- the purpose of insetting, stitching and trimming

- the information and materials required to meet job specification

- safety devices found on inline insetting-stitching-trimming machinery, their purpose and how to check they are functional

- how to set up and adjust inline insetting-stitching-trimming machinery to meet job requirements

- the causes of common faults and how to rectify them including stitch legs of unequal length, stitch legs not closed, stitches missing or marks on book spines

- the causes of faults with raw materials, processes and machinery used in your business and how to identify and treat them

- techniques for controlling quality including inspection, testing, sampling and use of input and output controls

- the impact that faults, in the process you are involved with, have on later processes and the quality of the end product

- types of problems that may need to be solved including machinery - electrical, mechanical, electronic, settings, component wear and tear, consumables needing replacement, materials - defects, shortages, incompatibility, systems, organisation and lack of skills or knowledge

- sources of information for solving problems including manufacturer's documentation / troubleshooting guides, colleagues, tutors / trainers / mentors and reference material – in house or external, such as the internet

- techniques for assessing machine faults including observation, listening, inspection of product, reports from colleagues / log reports, touch or smell (if safe to do so) and testing, such as electrical, mechanical, electronic

- the types of paper, board and other commonly used substrates including commonly used uncoated, coated, embossed papers and boards

- the grammage, thickness, opacity, brightness/whiteness, strength, dimensional stability, gloss of paper, board and other commonly used substrates

- how to maintain the quality of materials and protect them from damage, humidity and temperature during storage and handling

- how to label and identify materials

Developed by

Improve

Version Number

3

Date Approved

26 Feb 2021

Indicative Review Date

31 Jan 2024

Validity

Current

Status

Original

Originating Organisation

NSAFD

Original URN

PROPF352

Relevant Occupations

Printers, Printing Machine Minders and Assistants, Printing Trades

Suite

Post Press

Keywords

post press, print finishing, machinery, in-line, insetting, stitching, trimming