| COSVR810 Install and form specialist masonry elements |  |
| --- | --- |

Overview

This standard is about ​installing and forming specialist masonry support elements, interpreting information, adopting safe, healthy and environmentally responsible work practices, and selecting, preparing and using materials, components, tools and equipment

This standard is for people working in the occupational area of Trowel Occupations and can be used by operatives, supervisors and managers

Performance criteria

You must be able to:

P1 interpret the given information relating to the work and resources to confirm its relevance

P2 comply with the given, relevant legislation and official guidance to carry out your work and maintain safe and healthy work practices

P3 select the required quantity and quality of resources for the methods of work

P4 comply with organisational procedures to minimise the risk of damage to the work and surrounding area

P5 comply with the given contract information to carry out the work efficiently to the required specification

P6 complete the work within the allocated time, in accordance with the programme of work

Knowledge and understanding

You need to know and understand:

​

Performance Criteria 1

Interpretation of information

K1 the organisational procedures developed to report and rectify inappropriate

information

and unsuitable

resources

, and how they are implemented

K2 the types of

information

, their source and how they are interpreted

K3 the organisational procedures to solve

problems

with the

information

and why it is important they are followed

Performance Criteria 2

Safe work practices

K4 the level of understanding operatives must have of

information

for relevant, current

legislation and official guidance

and how it is applied

K5 how

emergencies

should be responded to and who should respond

K6 the organisational

security procedures

for tools, equipment and personal belongings

K7 what the accident reporting procedures are and who is responsible for making the report

K8 why, when and how

health and safety control equipment

should be used

K9 how to comply with environmentally responsible work practices to meet current

legislation and official guidance

Performance Criteria 3

Selection of resources

K10 the characteristics, quality, uses, sustainability, limitations and defects associated with the

resources

and how defects should be rectified

K11 how the

resources

should be used and how any

problems

associated with the

resources

are reported

K12 the organisational procedures to select

resources

, why they have been developed and how they are used

K13 the

hazards

associated with the

resources

and

methods of work

and how they are overcome

Performance Criteria 4

Minimise the risk of damage

K14 how to

protect work

from damage and the purpose of protection

K15 why

disposal of waste

should be carried out safely and how it is achieved

Performance Criteria 5

Meet the contract specification

K16 how

methods of work

, to meet the specification, are carried out and problems reported

K17 how

maintenance

of tools and equipment is carried out

Performance Criteria 6

Allocated time

K18 what the

programme

is for the work to be carried out in the estimated, allocated time and why deadlines should be kept

Scope/range

​

Performance Criteria 1

1 interpretation of drawings, specifications, schedules, method statements, risk assessments and manufacturers' information related to the work to be carried out

Performance Criteria 2

2 avoidance of risk by complying with the given information relating to at least four of the following

2.1 methods of work

2.2 safe use of health and safety control equipment

2.3 safe use of access equipment

2.4 safe use, storage and handling of materials, tools and equipment

2.5 specific risks to health

Performance Criteria 3

3 selection of resources associated with own work

3.1 materials, components and fixings

3.2 tools and equipment

Performance Criteria 4

4 protection of the work and its surrounding area from damage

5 maintain a clear and tidy work space6 disposal of waste in accordance with current legislation

Performance Criteria 5

7 demonstration of work skills to ​position, level, plumb, adjust and fix

8 use and maintain hand and power tools, and equipment

9 install and/or form fire barriers and/or breaks and support angles plus at least two of the following specialist masonry support elements to given working instructions

9.1 brick soffit systems

9.2 channel systems

9.3 wind posts

9.4 vapour and/or moisture barriers

9.5 wall starter kits

Performance

Criteria 6

10 completion of own work within the estimated, allocated time to meet the needs of other occupations and/or client

Scope/range related to knowledge and understanding

​

Disposal of waste

1 environmental responsibilities, organisational procedures, manufacturers' information, statutory regulations and official guidance

Emergencies

2 operative's response to situations in accordance with organisational authorisation and personal skills when involved with

2.1 fires, spillages, injuries

2.2 emergencies relating to occupational activities

Hazards

3 those identified by risk assessment, methods of work, manufacturers' technical information, statutory regulations and official guidance

Health and safety control equipment

4 identified by the principles of prevention for occupational use, types and purpose of each type, work situations and general work environment

4.1 collective protective measures

4.2 personal protective equipment (PPE)

4.3 respiratory protective equipment (RPE)

4.4 local exhaust ventilation (LEV)

Information

5 drawings, specifications, current legislation, schedules, method statements, risk assessments, manufacturers' information, oral and written instructions, sketches, electronic data,  official guidance and current regulations associated with installing and forming specialist masonry support elements

Legislation and official guidance

6 this relates to the operative's responsibilities regarding potential accidents, health hazards and the environment whilst working in the workplace, below ground level, in confined spaces, at height, with tools and equipment, with materials and substances, with movement and storage of materials by manual handling and mechanical lifting

Maintenance

7 operative care of hand and power tools, and equipment

Methods of work

8 application of knowledge for safe and healthy work practices, procedures and skills relating to the method and area of work and materials used to

8.1 identify the types, uses and characteristics of specialist masonry support elements; brick soffit systems, support angles, fire barriers and breaks, wind posts and wall starter kits

8.2 position, level, plumb, fix and integrate brick soffit systems

8.3 install and adjust masonry support angles

8.4 install and maintain the integrity of fire barriers and breaks

8.5 form and maintain the integrity of cavities

8.6 position and secure wall ties including spacing, particularly around openings

8.7 position and fix damp-proof barriers, cloak systems and cavity trays

8.8 form and install weep holes and vents

8.9 position bond and tape insulation materials

8.10 install wind posts

8.11 use wall starter kits

8.12 recognise and determine when specialist skills and knowledge are required and report accordingly

8.13 identify and follow the installation quality requirements

8.14 work with, around and in close proximity to plant and machinery

8.15 use hand and `power tools, and equipment

8.16 work at height

8.17 use access equipment

9 team work and communication

10 needs of other occupations associated with the installation and formation of specialist masonry elements

Problems

11 those arising from information, resources and methods of work

11.1 own authority to rectify

11.2 organisational reporting procedures

Programme

12 types of productivity targets and time scales

13 how times are estimated14 organisational procedures for reporting circumstances which will affect the work programme

Protect work

15 protect work against damage from general workplace activities, other occupations and adverse weather conditions

Resources

16 materials, components and equipment relating to types, quantity, quality, sizes and the sustainability of standard and/or specialist

16.1 specialist masonry support elements

16.2 fittings and fixings

16.3 hand and power tools, and equipment

17 confirm resources and materials conform with the specification

18 methods of calculating quantity, volume, length, width, area and wastage of materials associated with the method and procedure to install and form specialist masonry  elements

Security procedures

19 site, workplace, company, operative and vehicles

Developed by

Construction Skills

Version Number

1

Date Approved

11 Feb 2019

Indicative Review Date

11 Feb 2022

Validity

Current

Status

Original

Originating Organisation

ConstructionsSkills

Original URN

COSVR810

Relevant Occupations

Bricklayers and Mason

Suite

Trowel Occupations (Construction)

Keywords

Brick soffit systems; Support angles; Fire barriers and breaks; Wind posts; Wall starter kits