| IMPDY107 Control processing to produce ice cream |  |
| --- | --- |

Overview

​This standard covers the skills and knowledge needed for you to produce ice cream in food and drink manufacture and/or supply operations. Knowing how to produce the correct specification of ice cream is important to the production of a final product that adheres to regulatory food safety and organisational quality and yield specifications.

The raw material could be from a variety of sources (e.g. milk from sheep, goat, cow and buffalo).

You will need to know how to prepare equipment to produce ice cream and control the ice cream making process. You must also know how to adhere to regulatory standards and standard operating procedures. You must be able to apply your skills and knowledge within food safety regulations and any relevant international industry codes especially when referring to HACCP, TACCP and VACCP.

This standard is for you if you work in food and drink manufacture and/or supply operations and are involved in producing ice cream.

Performance criteria

You must be able to:

1. prepare relevant upstream pasteurisation, separation, mixing/blending and

standardisation equipment for efficient preparation of milk and raw materials

to produce the ice cream recipe

2. prepare to control ice cream processing equipment and associated

compliance documentation

3. use the correct personal protective equipment

4. check organisational standard operating procedures and product recipes

relating to the production of ice cream

5. check that all necessary plant, equipment, plans for production, ingredients

and services are available and fit for use

6. check documentation to ensure plant is ready for use

7. start up and control dairy processing equipment within the limits of your

responsibility

8. work at a pace suitable to meet the production plan

9. make adjustments to equipment to maintain organisational product

specifications

10. take representative samples for testing adhering to organisational sampling

plan within the limits of your responsibility

11. evaluate faults for impact on product and processing equipment operations

within the limits of your responsibility

12. make dairy product available to next stage of processing or packaging

13. prepare for changeover of product recipe or packaging in line with

organisational requirements

14. prepare to handover process on shift changeover in line with organisational

requirements

15. deal with product waste in line with legislation and

organisational requirements

16. liaise with relevant personnel to prepare plant for shutdown

17. process and route interphase product

18. establish and maintain effective spoken, written and electronic communication with

relevant people

19. complete the necessary documentation and process to organisational

requirements

Knowledge and understanding

You need to know and understand:

1. the food safety aspects of the production of ice cream and related products

including HACCP, TACCP and VACCP

2. the dairy science underpinning ice cream production

3. the health and safety aspects associated with the process

4. how ice cream is classified and the key differences between dairy ice cream,

cream ice and milk ice

5. the regulations relating to the composition requirements for different types of

ice cream

6. the regulations relating to the heat treatment and storage of ice cream for

sale

7. organisational specifications and equipment standard operating procedures

relating to ice cream production

8. the effect of incorporating different amounts of air on how light the final

product tastes

9. the key differences between equipment and processes in large and small

scale craft production ice cream producing environments.

10. the effect of heat treatment, homogenisation, cooling, ageing and freezing

on the final ice cream product quality and yield

11. the effect of mix composition, the amount of air incorporated (overrun) into

the ice cream and the type of freezer on the freezing temperature

12. why freezing must take place quickly and its effect on ice crystal

formation and the smoothness of the final ice cream product

13. the term "overrun" and its importance to ice cream specifications, quality

yield and the economic value of the final ice cream product

14. why it is important to control storage temperature of ice cream

15. common faults found in ice cream and how they occur

16. how to maintain communication with relevant people when producing ice

cream

17. how to control product temperature when storing and processing dairy

products and ingredients used when making ice cream

18. why and how to control equipment to ensure the ice cream meets

organisational final product quality, yield and productivity specifications

19. how to adjust ice cream processing in response to product quality tests

20. what the actions are to address variations in intake milk/whey fat levels,

type of fat, environmental factors including seasonality and temperature

21. common sources of contamination during processing, how to avoid these

and what might happen if this is not done

22. how to deal with product that can be re-cycled or re-worked

23. the hygienic design of ice cream plant

24. how the manufacture process and support operations  use digital technologies rather than paper based systems to monitor and control production

Developed by

Improve

Version Number

3

Date Approved

30 Mar 2023

Indicative Review Date

30 Mar 2028

Validity

Current

Status

Original

Originating Organisation

Improve

Original URN

IMPDY107

Relevant Occupations

Food Preparation Trades, Manufacturing Technologies, Process Operatives, Process, Plant and Machine Operatives

Suite

Dairy

Keywords

Dairy; ice; cream; produce; food; drink; yield; quality; ingredients