| IMPPO236 Control bottle washing in food and drink operations |  |
| --- | --- |

Overview

​

T

his standard is about controlling bottle-washing in food and drink

operations.

It details the skills and knowledge required to start up, run and shut down

equipment, as well as being able to take the appropriate action should

operating problems occur. It is also about working to production

schedules. Bottle-washers are designed to clean used glass bottles that

are to be recycled. They are complex items of equipment that rely on

correct maintenance of liquid levels, detergent concentrations and

temperature profiles. The bottle-washer environment can be hazardous

and correct Personal Protective Equipment (PPE), especially eye

protection, should always be worn.

This standard is for you if you are involved in washing returnable glass

bottles in automatic bottle-washer equipment.

Performance criteria

You must be able to:

​1.

prepare for bottle washing according to the legal or regulatory requirements,

the organisational health and safety, hygiene and environmental standards

and instructions

2. check washer tank levels, temperatures and chemical concentrations

3. set up equipment according to specification

4. check that bottles, crates, cases and product are available and fit for use

5. check that services meet requirements

6. start up the plant and check that it is running to specification

7. take effective action in response to operating problems

8.

maintain effective communication

9. carry out bottle washing according to the legal or regulatory requirements,

the organisational health and safety, hygiene and environmental standards

and instructions

10.

use equipment and check that it is supplied with empty bottles that are not

grossly contaminated

11. check for any caustic carry-over using approved methods

12. achieve the required throughput

13.

take effective action safely in response to operating problems within the

limits of your responsibility

14.

finish bottle washing according to the legal or regulatory requirements, the

organisational health and safety, hygiene and environmental standards and

instructions

15. follow procedures to shut down equipment

16. deal with items that can be re-cycled or re-worked

17. return surplus materials and consumables to the correct place at the end

Knowledge and understanding

You need to know and understand:

1. what

the legal or regulatory requirements, the organisational health and

safety, hygiene and environmental standards and instructions are and what

may happen if they are not

2.

the purpose and importance of bottle washing

3. what equipment and tools to use and their correct condition

4. what materials to use and in what quantity

5. how to obtain and interpret the relevant process or ingredient specification

6. what recording, reporting and communication is needed and how to carry

this out and the reasons why it is important to do so

7. what action to take when the process specification is not

8.

how to carry out the necessary pre start checks and why it is important to do

so

9. how to follow the start up procedures for the process and why it is important

to do so

10. how to obtain the necessary resources for the bottle washing

11. how to follow work instructions and why it is important to do so

12. common sources of contamination during processing, how to avoid these

and what might happen if this is not done

13. how to operate, regulate and shut down the relevant equipment

14. when it is necessary to seek assistance and how to seek it

15. how to follow the relevant process control procedures and why it is

important to do so

16. different ways to carry out bottle washing

17. how to carry out the process in an efficient manner and why it is important

to do so

18. what the limits of your own authority and competence are and why it is

important to work within them

19. how to dispose of waste

20. how to make equipment ready for future use