| IMIEV02a Provide a first response to a broken-down or accident damaged electric vehicle |  |
| --- | --- |

Overview

This standard is designed for those who provide a first response to a broken-down or accident damaged electric vehicle, for example, those working for the emergency services and roadside recovery operators; the standard would also be appropriate for those involved in the dismantling and disposal of electric vehicles. It covers the working practices and knowledge needed to carry out a risk assessment and work safely around an electric vehicle that may have damage to its high and/or low voltage systems.

For the purposes of this standard, an electric vehicle is any vehicle that is in part or wholly electrically propelled. This would include:

- Hybrid (HEV) - to include mild/micro hybrid vehicles where the voltage is considered dangerous.

- Plug-in Hybrid (PHEV)

- Extended Range Electric Vehicle (ER-EV) or Range Extended Electric Vehicle (RE-EV)

- Battery Electric Vehicle (BEV) or Pure Electric Vehicle (PEV)

- Fuel Cell Electric Vehicle (FCEV).

This standard does not deem someone competent to maintain, service or repair an electric vehicle’s high voltage systems and their components.

Performance criteria

You must be able to:

P1 Identify the electric vehicle type and collect relevant information about the

vehicle

and associated

hazards

P2 Wear personal protective equipment (PPE) and use safety equipment in line with your working procedures and appropriate to the work activities you are carrying out.

Please note, this does not include specific PPE for working on the high voltage system.

P3 Carry out a dynamic risk assessment to identify the potential

hazards

presented by the electric vehicle, assess the risks and formulate an agile mitigation plan

P4 Follow the correct procedures to make the

vehicle

safe prior to starting any work activities. If the procedure to do this is NOT within your level of training, competence and authority, refer to the relevant person.

P5 Carry out work activities in a way that minimises risks to yourself, other people and the environment

P6 Recognise when the

vehicle

has problems you cannot deal with yourself, refer to a relevant person and follow their instructions

P7 Record and report the work activities you have carried out on or near the

vehicle

to relevant person.

P8 Follow a robust, documented/logged handover procedure

Knowledge and understanding

You need to know and understand:

Use of technical information

K1 How to identify an electric

vehicle

and its type

K2 How to find, interpret and use sources of information applicable to electric vehicles as appropriate to your job role

K3 How to identify high voltage electrical components in an electric vehicle

Legislative and organisational requirements and procedures

K4 The health and safety legislation, industry codes of practice or guidelines and workplace procedures relevant to working on, near or with electric vehicles, and the safety of the working environment

K5 How to carry out a dynamic risk assessment on damaged and broken-down electric vehicles

K6 How to formulate an agile plan to mitigate any risks

K7 The manufacturer’s and your workplace procedures for:

K7.1 assessing and managing the risks associated with damaged and broken down electric vehicles

K7.2 identifying the

indicators

associated with unstable electric vehicles

K7.3 ensuring that the

vehicle

has been made safe as appropriate to the work you are carrying out, within your level of training, competence and authority

K7.4 referring/reporting problems when working with electric vehicles, including when the actions required fall outside of your level of responsibility/authority

K7.5 recording and reporting work carried out on electric vehicles

K8 Organisational procedures that must be followed in the event of electric shock

K9 The procedure to follow if the situation includes electric vehicle supply/charging equipment (EVSE)

K10 The importance of adhering to a robust, documented handover procedure

High voltage component construction and layout

K11 The fundamental features and principles of high voltage components, including battery modules, electric motors, associated components and auxiliary systems

K12 How to identify the location of high voltage cables and components, for example, by labelling and colour

K13 The fundamental features and principles of alternative fuel sources and systems on electric vehicles, including hydrogen fuel cells

Vehicle system operation

K14 The main differences between an electric vehicle and a non-electric vehicle and its basic operation, including regeneration

K15 The charging systems associated with electric vehicles and how to use them safely, including the use of plug-in charging equipment

K16 The specific manufacturer’s guidelines and the precautions necessary when charging, connecting an auxiliary power source to or moving an electric vehicle

K17 How to safely mobilise/relocate an electric vehicle

K18 How to safely and effectively immobilise an electric vehicle within your level of authority/competency

K19 When

vehicle

systems may self-operate

Hazards associated with electric vehicles

K20 The

hazards

associated with high and low voltage systems including batteries and other high voltage electrical vehicle components

K21 The implications and effects of electricity through the human body

K22 The

hazards

associated with alternative fuel systems, including hydrogen fuel cells

K23 The

hazards

associated with electric vehicles when exposed to extreme temperatures, impact and other adverse conditions

K24 The

indicators

of thermal runaway

K25 The implications and

hazards

of remote

vehicle

control

Scope/range

- Vehicle

- - any vehicle that is in part or wholly electrically propelled. This would include

- 1.1. Hybrid (HEV) - to include mild/micro hybrid vehicles where the voltage is considered dangerous.

- 1.2. Plug-in Hybrid (PHEV)

- 1.3. Extended Range Electric Vehicle (ER-EV) or Range Extended Electric Vehicle (RE-EV)

- 1.4. Battery Electric Vehicle (BEV) or Pure Electric Vehicle (PEV)

- 1.5. Fuel Cell Electric Vehicle (FCEV)

- Hazards

- may include:

- 2.1. High voltage

- 2.2. Unpredictable vehicle movements

- 2.3. Gases

- 2.4. High-voltage system residual charge

- 2.5. Highly magnetic components

- 2.6. Fuel explosion

- 2.7. Hazardous materials

- 2.8. Leaking electrolytes

- 2.9. Pressurised systems

- Indicators

- may include:

- 3.1. Visual vehicle warning alerts

- 3.2. Audible vehicle warning alerts

- 3.3. Smoke or vapour emission

- 3.4. ‘Popping’, ‘crackling’ or ‘hissing’ sound

- 3.5. Burning smell

- 3.6. Strong odour

- 3.7. Physical impact damage

Glossary

Additional Information

This section contains examples and explanations of some of the terms used but does not form part of the standard.

Dynamic risk assessment

- the practice of continually mentally observing, assessing and analysing an environment while working, to identify and remove risk. The process allows individuals to identify a hazard on the spot and make quick decisions in regard to their own safety.

Handover procedure

– to include mark up of vehicle and detailed incident record, including hazards encountered and actions taken.

Hazards associated with high voltage electrical vehicle components

- exist not only during work on high voltage systems, as specified above, but also on all other high-power electrical drive systems and high-pressure storage systems. Vehicle and equipment manufacturers’ guidance should be followed at all times.

High voltage

– Regulation No 100 of the Economic Commission for Europe of the United Nations (UNECE) — Uniform provisions concerning the approval of vehicles with regard to specific requirements for the electric power train, states that: ‘High Voltage’ means the classification of an electric component or circuit, if its working voltage is > 60 V and ≤ 1 500 V DC or > 30 V and ≤ 1 000 V AC root mean square (rms). Electricity at Work Regulations (1989), and associated HSE guidance should be followed at all times.

N.B. Some electric vehicles may operate at voltages below or above industry recognised standards.

Relevant person

- May be someone within your organisation, or other authority.

Relocate

- Minimal movement to facilitate a rescue.

Sources of information applicable to electric vehicles

- Examples include manufacturer guidance, hard copy manuals, online data and data obtained from on- board diagnostic displays. It may also include control room or other personnel, the vehicle driver or passengers.

Work activities – dealing with electric vehicles in the course of an emergency first response or recovery.

Developed by

IMI

Version Number

4

Date Approved

31 Mar 2024

Indicative Review Date

01 Apr 2027

Validity

Current

Status

Original

Originating Organisation

IMI

Original URN

IMIEV2

Relevant Occupations

Firefighters, Police, Roadside Assistance Manager, Roadside Assistance Senior Operator, Supervisory Recovery Technical Operator, Vehicle Recovery Operator, Vehicle Recovery Technical Operator, Vehicle Recycler, Vehicle Dismantler, Emergency First Responders

Suite

Electric and Hybrid Vehicles

Keywords

Electric Vehicle; recovery; assistance; breakdown; accident; damage