| COSVR450 Install cavity wall insulation |  |
| --- | --- |

Overview

"This standard is about preparing and installing cavity wall insulation, interpreting information, adopting safe, healthy and environmentally responsible work practices, selecting and using materials, components, tools and equipment

This standard is for people working in the occupational area of insulation and building treatments and can be used by installers, supervisors and managers"

Performance criteria

You must be able to:

"P1  interpret the given design information relating to the work and resources to confirm its accuracy, completeness and relevance to the building type, fabric and condition for the following:

1.1   drawings

1.2   specifications

1.3   schedules

1.4   method statements

1.5   risk assessments

1.6   manufacturers’ information

1.7  data sheets

P2  comply with the given, relevant legislation, standards and official guidance to carry out your work and maintain safe and healthy work practices relating to the following:

2.1   methods of work

2.2   safe use of health and safety control equipment

2.3   safe use of access equipment and harness systems

2.4   safe use, storage and handling of materials,

tools and equipment

2.5   operative maintenance of installation equipment

2.6   specific risks to health including mental health

2.7   specific risks associated with ventilation (roof space, inside the

property and under floor) and combustion appliances

P3   select the required quantity and quality of resources for

the methods of work

3.1   check the suitability, compatibility and characteristics of

the materials, components and finishes and determine

if they are moisture open or moisture closed and their impact

on the building

3.2   record and report issues or defects

3.3   select tools and equipment

P4   comply with organisational procedures to minimise the risk

of damage to the work and surrounding area by:

4.1   protecting the work and its surrounding area from damage

4.2   maintaining a safe, clear and tidy work area

4.3   disposing of waste in accordance with current legislation

P5 comply with the given contract information to carry out the work

efficiently to the required specification by the following:

5.1 demonstrate work skills to measure, mark out, calibrate,

monitor, fit and fill, make good

5.2 carry out external and internal pre-installation check, assessing,

recording and reporting issues to include:

- suitable access

- property suitability

- structural integrity

- dampness

- decay

- exposure ratings

- vents and ventilation

- services (gas, electric, water, media cables)

5.3 check, record and report issues with construction ventilation,

flues, chimneys and combustion air ventilators pre and post

installation

5.4 use and maintain all work tools and installation equipment

5.5 drill holes to specified patterns using depth gauges and right

angled drilling only, selecting the correct masonry drill bit,

speed and setting, and taking effective steps to minimise the

impact to the building fabric and preventing rubble falling into

the cavity

5.6 fit cavity barriers

5.7 assemble and operate installation equipment, measuring

density, flow and quality tests

5.8 prepare for and install cavity wall insulation to given system

designer specification, method statement and the required

standard

5.9 fill holes with matching and suitable materials

5.10 clean, disassemble and prepare installation processing

equipment for transportation

5.11 handover and sign off to the customers satisfaction

5.12 carry out post installation checks

P6   complete your work within the estimated, allocated time

and performance requirements of the system design, method

statement and the required standard"

Knowledge and understanding

You need to know and understand:

"Performance Criteria 1

Interpretation of information

K1.  why organisational procedures have been developed and how

they are implemented

K2.  types of information, their source, accuracy, completeness

and how they are interpreted in relation to:

2.1   drawings

2.2   specifications

2.3   schedules

2.4   method statements

2.5   risk assessments

2.6   design

2.7   standards

2.8   manufacturers' information

2.9   data sheets

2.10  official guidance

2.11  current legislation and regulations governing buildings

K3. the importance of organisational procedures to solve problems

and why it is important to follow them

Performance Criteria 2

Safe work practices

K4. the relevant, current legislation, standards and official guidance

and how they are applied

K5. the types of fire extinguishers and how and when they are used

in relation to water, CO2,foam, powder

K6. how emergencies should be responded to in accordance with

organisational authorisation and personal skills in relation to:

6.1   fires, spillages, injuries

6.2   emergencies relating to occupational activities

6.3   identification of and reporting of asbestos containing materials

K7. the organisational security procedures for tools, equipment and

personal belongings in relation to:

7.1   site

7.2   workplace

7.3   siting and location of vehicles

7.4   company

7.5   customer

7.6   access equipment

7.7   material and waste storage

7.8   the general public

K8.  how to report risks and hazards identified by the following:

8.1   risk assessment

8.2   personal assessment

8.3   methods of work

8.4   manufacturers' technical information

8.5   data sheets

8.6   statutory regulations

8.7   official guidance

8.8   Control of Substances Hazardous to Health (COSHH)

K9.  the accident reporting procedures and who is responsible for

making the report

K10. why, when and how health and safety control equipment

identified by the principles of prevention should be used in

relation to:

10.1   collective protective measures

10.2   personal protective equipment (PPE)

10.3   respiratory protective equipment (RPE)

10.4   local exhaust ventilation (LEV)

K11.  environmentally responsible work practices to meet current

legislation, standards and official guidance when dealing with

potential accidents, health hazards and the environment

in relation to:

11.1  below ground level

11.2  confined spaces

11.3  at height

11.4  tools and equipment

11.5  materials and substances

11.6  movement and storage of materials by manual handling and

mechanical lifting

Performance Criteria 3

Selection of resources

K12.  why the characteristics, compatibility, quality, uses,

sustainability, limitations and defects associated with the

resources are important and how defects should be rectified

K13.  the organisational procedures to select resources, why they

have been developed and how they are used

K14.  how to confirm the resources and materials conform with the

specification

K15.  how the resources should be used and how any problems

associated with the resources are  reported in relation to:

15.1  protective sheeting

15.2  warning signs

15.3  public protection equipment

15.4  calibration equipment

15.5  essential airway sleeves

15.6  cavity barriers

15.7  mortar mix

15.8  mortar dyes

15.9  insulation

15.10  combustion vents

15.11  all work tools

15.12  installation equipment

K16.  how to identify the hazards associated with the resources and

methods of work and how they are overcome

K17.  how to calculate the quantity of materials required and used

to ensure adeqaucy of fill as per the system designer

specification and wastage associated with the

method and procedure to install cavity wall insulation

Performance Criteria 4

Minimise the risk of damage

K18.  the importance of protecting the work and its surrounding

area against the risk of damage

K19.  how to protect work and its surrounding area from damage by

general workplace activities, other occupations and adverse

weather conditions and how to minimise damage to existing

building fabric

K20.  why and how the disposal of waste must be carried out safely

in accordance with the following

20.1   current legislation

20.2   environmental responsibilities

20.3   organisational procedures

20.4   manufacturers' information

20.5   data sheets

20.6   statutory regulations

20.7   official guidance

K21.  why it is important to maintain a safe, clear and tidy work

area

Performance Criteria 5

Meet the contract specification

K22.  how the methods of work to meet the specification, are carried

out and how problems are  identified and reported by the

application of knowledge for safe, healthy and

environmental  work practices, procedures and skills relating

to the method and area of work

22.1 the suitability, compatibility and characteristics of the materials,

components and finishes, and determine if they are moisture

open or moisture closed, their impact on the building and

their appropriateness to the design and physical application

22.2 how to record and report issues or defects with the materials,

components and finishes

22.3 why it is important to carry out external and internal pre-

installation checks

22.4 how to carry out external and internal pre-installation checks,

assessing, recording and reporting issues to include but not

limited to:

- suitable access

- property suitability

- structural integrity

- dampness

- decay

- exposure ratings

- vents and ventilation

- services (gas, electric, water, media cables)

22.5 why it is important to ensure that all necessary repairs are

completed prior to installation

22.6 how to recognise, record and report the key issues that may

inhibit commencement of the work including but not limited

to:

- condition of building fabric

- identification of any areas of potential water penetration

- visibility and completeness of damp proof course

- condition of window and door seals

- height of internal floors in relation to external floor height

- condition of roof

- damaged or spalled brickwork

- drainage and down pipes

- protection and existence of sub floor ventilation

- cavity width and identification of any debris

22.7 how to identify when specialist skills and knowledge are

required and report accordingly including but not limited to:

- fire safety

- electrical

- asbestos

- Radon

- heritage

-

architectural features

-

ecology

- ventilation

22.8 the relevance of an assessment of significance and how to

recognise specific requirements for structures of special

interest, traditional construction, hard-to-treat buildings and

historical significance

22.9 how to identify, record, report and rectify unintended

consequences not addressed in the design, including but not

limited to the existence of: thermal bridges, thermal bypassing

and water ingress, inadequate ventilation and condensation

risk

22.10 why it is important to avoid unintended consequences

22.11 why it is important to explain installation procedure to

building occupants to include but not limited to the following:

- scope and work programme

- safety requirements during the installation process

- protection of property and personal items

- specific benefits and implications to include homeowner

information

- agreed standards of making good

22.12 the implications of existing guarantees and warranties that

may be compromised by the installation to include but not

limited to:

- wall ties

- windows

- damp proof course (DPC)

- renders

- Tyrolean coatings

- silicone weather proof coatings

22.13 how to work with, around and in close proximity to plant and

machinery

22.14 how to direct and guide the operations and movement of

plant and machinery to ensure protection of a safe working

environment

22.15 how to identify and follow the installation quality

requirements

22.16 how and why it is important to check, record and report issues

with construction ventilation, flues, chimneys and combustion

air ventilators pre and post installation

22.17 why it is important to ensure pre-installation material checks

are within specified parameters, to include checking and

recording batch number and reporting defects

22.18 how to assemble and operate installation processing

equipment in line with manufacturers and system manuals

22.19 how to calibrate equipment to measure density, flow and

quality tests to ensure they are in line with manufacturers

specifications and material requirements

22.20 why effective selection of PPE equipment to avoid

cementation dust is important

22.21 how to drill holes to specified patterns and the importance of

using depth gauges and right angled drilling only, selecting

the correct masonry drill bit, speed and setting, and taking

effective steps to minimise the impact to the building fabric

and preventing rubble falling into the cavity

22.22 how to fit cavity barriers in accordance with specification from

roof to ground level in order to avoid overspill and underspill

between the two separated cavity elements

22.23 how to install cavity wall insulation from inside and outside of

a building including lance techniques

22.24 why it is important to ensure effective and safe operation of

equipment and consistency of fill using the appropriate

technique for the selected material (to include bead using

adhesive bonding agents and blown mineral wool)

22.25 how to fill holes with matching and suitable materials to

ensure evidence of the drill pattern is minimised and the finish

is in keeping with the original building texture and colour

22.26 why it is important to clean and disassemble installation

processing equipment and pack away for transportation

22.27 the different types of air and vapour control layers and

breather membranes, where and how they should be used and

why it is important to install them correctly

22.28 the importance of ensuring the integrity of air and vapour

control layers and breather membranes following installation

and the need to maintain continuity

22.29 why it is important to immediately record and report

unforeseen events including but not limited to equipment

malfunctions, situations and faults not identified in the

original design

22.30 why it is important to complete post installation checks in

accordance with the system designer installations operations

manual and report issues to include but not limited to

safeguarding the combustion ventilation and report defects

22.31 why it is important to provide post installation advice and

guidance to building occupants including homeowner packs

22.32 how to handover and sign off to the customers satisfaction

22.33 how to use all work tools and installation equipment in line

with manufacturers and system specifications

22.34 how to work at height using access equipment and harness

systems

22.35 how and why maintenance of all work tools and installation

equipment is carried out

K23. the importance of team work and communication

K24. the needs of other occupations associated installing insulation

to cavity walls

Performance Criteria 6

Allocated time

K25.  the programme work to be carried out including the

estimated and allocated time and why deadlines should be

kept

K26.  the types of progress charts, timetables and estimated times

and the organisational procedures for reporting circumstances

which will affect the work programme"

Developed by

CITB

Version Number

4

Date Approved

30 Aug 2020

Indicative Review Date

30 Aug 2025

Validity

Current

Status

Original

Originating Organisation

CITB

Original URN

COSVR450

Relevant Occupations

Construction Operatives

Suite

Insulation and Building Treatments (Construction)

Keywords

Cavity barriers; Mortar mix; Mortar dyes; Cavity elements; Airway sleeves