| SEMMME219 Operating CNC turning machines |  |
| --- | --- |

Overview

This standard identifies the competences you need to carry out turning operations, in accordance with approved procedures, using Computer Numerical Control (CNC) machines, or CNC machining centres. You will confirm with the machine setter that the machine is ready for the operations to be performed and that all the required components/materials and consumables are available. In operating the machine, you will be expected to follow the correct procedures for calling up the operating program, dealing with any error messages and executing the program activities safely and correctly.

You will be expected to produce a range of components that combine a number of different features, such as parallel, stepped and tapered diameters, drilled and reamed holes, internal and external threads, special forms and profiles. You will be required to operate the CNC machine in line with safe working practices and approved procedures, to monitor continuously the machining operations and, where necessary, make minor adjustments or seek the help of the setter to make the required adjustments, in order to ensure that the work output is to the required quality and accuracy. Meeting production targets will be an important issue, and your production records must show consistent and satisfactory performance.

Your responsibilities will require you to comply with organisational policy and procedures for the machining activities undertaken, and to report any problems with the machining activities that you cannot personally resolve, or are outside your permitted authority, to the relevant people. You will be expected to work to instructions, with a minimum of supervision, taking personal responsibility for your actions and for the quality and accuracy of the work that you produce.

Your underpinning knowledge will be sufficient to provide a sound basis for your work, and will enable you to adopt an informed approach to applying CNC machining procedures. You will have an understanding of the CNC turning process and its application, and will know about the equipment, materials and consumables in adequate depth to provide a sound background for carrying out the activities to the required specification.

You will understand the safety precautions required when working with the machine, its associated tools and equipment. You will be required to demonstrate safe working practices throughout, and will understand the responsibility you owe to yourself and others in the workplace.

Setting up of the machine, its programming, tooling and associated workholding devices, is the subject of another standard and is the responsibility of the machine-tool setter.

Performance criteria

You must be able to:

P1    work safely at all times, complying with health and safety and other relevant regulations, directives and guidelines

P2    confirm that the CNC turning machine is set up and ready for operation

P3    operate the machine controls in accordance with safe working practices and operational procedures

P4    deal with problems within your control and report those that cannot be solved

P5    operate the CNC turning machine to produce machined components

P6    monitor the computer process and ensure that the production output is to the required specification

P7    complete and store all relevant documentation in accordance with organisational requirements

P8    shut down the equipment to a safe condition on conclusion of the activities

Knowledge and understanding

You need to know and understand:

K1    how to work safely at all times, complying with health and safety and other relevant regulations, directives and guidelines

K2    the safety mechanisms on the CNC turning machine, and the procedure for checking that they function correctly

K3    the importance of wearing the appropriate personal protective equipment (PPE), and of keeping the work area clean and tidy

K4    the main features of the CNC turning machine, and the accessories that can be used

K5    the various CNC turning operations that can be performed, and the methods and equipment used

K6    the operation of the various hand and automatic modes of machine control

K7    how to stop the CNC lathe in both normal and emergency situations, and the procedure for restarting after an emergency

K8    how to use the visual display and understand the various messages displayed

K9    the function of error messages, and what to do when an error message is displayed

K10    how to find the correct restart point in the program when the machine has been stopped before completion of the program

K11    where to obtain the component drawings, specifications and/or job instructions required for the components to be machined

K12    how to extract and use information from engineering drawings or data and related specifications in relation to work undertaken

K13    how to use imperial and metric systems of measurement

K14    the application of roughing and finishing cuts, and the effect on tool life, surface finish and dimensional accuracy

K15    the application of cutting fluids with regard to a range of different materials

K16    the effects of clamping the work piece in a chuck/work holding device, and how this can cause distortion in the finished components

K17    how to recognise CNC turning faults, and how to identify when tools need re-sharpening/replacing

K18    the quality control procedures that are used, inspection checks to be carried out, and the equipment that will need to be used

K19    the issues that can occur with the CNC turning activities, and how these can be overcome

K20    the extent of your own responsibility and to whom you should report if you have problems that you cannot resolve

Scope/range related to performance criteria

1.    Confirm that the machine is ready for operation by checking all of the following:

1.1    obtain and use the appropriate documentation (such as job instructions, drawings, quality control documentation)

1.2    adhere to procedures or systems in place for risk assessment, personal protective equipment and other relevant safety regulations and procedures to realise a safe system of work

1.3    confirm with the machine setter that the machine is ready for production

1.4    where appropriate, seek any necessary instruction/training on the operation of the machine

1.5    ensure that machine guards are in place and are correctly adjusted

1.6    hold components securely, without distortion

1.7    check that the operating program is at the correct start point and the work piece is clear of the machine spindle

1.8    follow the defined operating procedures and apply safe working practices and procedures at all times

1.9    ensure that machine settings are adjusted as and when required (either by yourself or the setter) to maintain the required accuracy

1.10    ensure that the components produced meet the required specification for quality and accuracy

1.11    leave the work area and machine in a safe and appropriate condition on completion of the activities

2.    Operate one of the following CNC turning machines:

2.1    CNC lathe

2.2    CNC machining centre

3.    Produce machined components which combine different operations and cover six of the following features:

3.1    parallel diameters

3.2    internal profiles

3.3    eccentric diameters

3.4    stepped diameters

3.5    external profiles

3.6    external screw threads

3.7    tapered diameters

3.8    reamed holes

3.9    internal screw threads

3.10    flat faces

3.11    tapped holes

3.12    chamfers and radii

3.13    internal undercuts

3.14    drilled holes

3.15    bored holes

3.16    external undercuts

3.17    parting-off

3.18    grooves

3.19    other specific feature

4.    Machine one of the following types of material:

4.1    ferrous

4.2    non-ferrous

4.3    non-metallic

5.    Use appropriate gauges or instruments to carry out the necessary checks, during production, for accuracy of three of the following:

5.1    diameters

5.2    lengths/depths

5.3    hole size/fit

5.4    surface finish

5.5    angle/taper

5.6    thread fit

6.    Produce components with dimensional accuracy, form and surface texture within all the relevant quality and accuracy standards, as applicable to the operations performed:

6.1    dimensional tolerance equivalent to relevant standard

6.2    surface finish 63 µin or 1.6 µm

6.3    reamed and bored holes within H8

6.4    angles within +/- 0.5 degree

6.5    screw threads BS medium fit

Developed by

Enginuity

Version Number

3

Date Approved

31 Mar 2026

Indicative Review Date

01 Apr 2029

Validity

Current

Status

Original

Originating Organisation

Enginuity

Original URN

SEMMME2-19

Relevant Occupations

Engineering, Engineering and Manufacturing Technologies, Engineering Technicians

Suite

Mechanical Manufacturing Engineering Suite 2

Keywords

engineering; manufacturing; mechanical; machining; operating; CNC; lathes; turning; machine centres; profile turning