| UKFTTCS12 Remove stains on textile items through cleaning processes |  |
| --- | --- |

Overview

​This standard is for those who inspect items and detect stains and use chemicals or detergents to remove stains. This may be for items being wet or dry cleaned.

This standard is for textile care operatives who specialise in stain removal.

Performance criteria

You must be able to:

​P1    prepare the work station for stain treatment and/or removal, item inspection and pre-spotting

P2    identify fabric fibres and the correct method for removing stains from the item prior to treatment

P3    identify staining type or possible type using feel, odour, colour, appearance and location

P4    test delicate items and accessories before treatment

P5    ensure delicate items and accessories are protected during treatment procedures

P6    correctly identify wet side and dry side staining

P7    correctly identify protein, albumen and tannin staining

P8    test the item to check for colour fastness to spotting reagents and chemicals

P9    apply and treat the item using wet side and dry side proprietary chemicals

P10    select and use the correct method to remove stains from items without damaging the fabric

P11    safely flush out applied chemicals or detergents from the item

P12    safely handle contaminated, blood stained or unpleasant soiled items in line with organisational procedures

P13    inspect the item following stain removal to ensure it meets organisational quality standards

P14    ensure the work station is cleaned and prepared correctly between treatments

Knowledge and understanding

You need to know and understand:

​K1    how to operate stain removal equipment

K2    the difference between cold spotting and steam spotting tables

K3    the importance of using dedicated pre-spotting tables

K4    why it is important to maintain the cleanliness of the work station between treatments

K5    the correct steam, water, air pressure and lighting requirements to carry out stain removal

K6    the requirements of ventilation air extraction during stain removal processing

K7    the differences between; absorbed staining, built up staining and compound staining

K8    the objectives and limitations of stain removal

K9    the definition of pre-spotting

K10    the risks associated with the dry-cleaning of items containing water after wet side removal treatments or slab spotting

K11    how to interpret care labels

K12    why certain stains need additional treatment or special precautions

K13    the properties of main fibres used in fabrics

K14    the problems that may arise when different fabric/fibre types and construction, trimmings and accessories are brought into contact with certain chemicals

K15    the tests used to identify fabrics and fibres

K16   the different methods used for stain removal including: mechanical action, dissolving , softening and lubrication, digestion and chemical reaction

K16    correct use of stain removal chemicals, their limitations and other stain removal techniques

K17    the personal protection measures to follow when using hazardous chemicals

K18    how to identify hazardous substances

K19    how to safely dispose of hazardous substances following use

K20    the organisational quality standards for stain removal

K21  statutory responsibilities under health, safety and environmental legislation and regulations during the stain treatment process

Developed by

UKFT

Version Number

2

Date Approved

30 Mar 2023

Indicative Review Date

30 Mar 2026

Validity

Current

Status

Original

Originating Organisation

Creative Skillset

Original URN

SKSLWDC12

Relevant Occupations

Textile and Garment Trades, Laundry Operative, Wet Cleaner, Dry Cleaner

Suite

Textile Care Services

Keywords

Laundry; Wet cleaning; Dry cleaning; Staining; Spotting;Chemicals; Safety; Fabrics; Fibres;