| SEMFWE230 Joining materials by resistance spot welding |  |
| --- | --- |

Overview

This standard identifies the competencies you need to set up and use portable, and simple fixed spot welding machines, in accordance with approved instructions or welding procedures. You will be expected to check that the equipment is fit for purpose, that electrodes are correctly profiled, and that the component parts are in the correct condition for spot welding. In preparing the equipment, you will need to set the welding current, welding and squeeze times and electrode pressure. You must operate the equipment safely and correctly, and make any necessary adjustments to the equipment settings and parameters within permitted tolerances, in order to achieve a weld quality and tolerances that meet the specification.

Your responsibilities will require you to comply with organisational policy and procedures for the welding activities undertaken, and to report any problems or adjustments to the equipment that you cannot resolve, or are outside your permitted authority, to the relevant people. You will be expected to work to instructions, with a minimum of supervision, taking personal responsibility for your own actions and for the quality and accuracy of the work that you carry out.

Your underpinning knowledge will be sufficient to provide a sound basis of your work, and will enable you to adopt an informed approach to applying spot welding procedures and instructions. You will understand how the resistance spot welding process works, and will know about the equipment, materials and consumables, in adequate depth to provide a sound background to the process operation and for carrying out the activities to the required specification.

You will understand the safety precautions required when working with the resistance spot welding equipment. You will be required to demonstrate safe working practices throughout, and will understand the responsibility you owe to yourself and others in the workplace.

Performance criteria

You must be able to:

- work safely at all times, complying with health and safety and other relevant regulations, directives and guidelines

- follow the relevant joining procedure and work instructions

- confirm that the machine is set up and operating correctly, ready for the joining operations to be carried out

- check that the parent material, components, consumables and joint preparation comply with specifications

- carry out and monitor the machine operations in accordance with specifications and job instructions

- achieve joints of the required quality and specified dimensional accuracy

- make sure that the rate of output is as specified

- deal promptly and effectively with problems within your control and report those that you cannot solve

- shut down the equipment to a safe condition on conclusion of the joining activities

Knowledge and understanding

You need to know and understand:

- the specific safety precautions to be taken when operating resistance welding equipment (working with machinery, the use of appropriate personal protective equipment (PPE); the use of safety screens; operation of machine safety devices; closing down the machine on completion of the welding activities; statutory requirements, risk assessment procedures and relevant requirements of HASAWA, COSHH and Work Equipment Regulations; safe disposal of waste materials)

- the hazards associated with resistance welding equipment (dangers from live internal electrical components, fumes, hot metal, expulsion of hot particles, moving parts of machines), and how they can be minimised

- the basic principles of resistance welding; heat and pressure to form a weld; heating effect of welding current; principal features of the welded joint; heat input; welding and pressure cycles; terminology used in welding

- the key components and features of the equipment used (power source; welding head; power range; electrical parameters (such as arc voltage, current, electrode pressure and welding time); systems for parameter control; how variation in the parameters influence weld features, quality and output)

- extracting information required from drawings and welding procedure specifications(to include symbols and conventions to appropriate BSEN or ISO standards in relation to the work undertaken)

- operation of the machine controls and their function; equipment care procedures

- monitoring the equipment during the welding process; fine tuning parameters to maintain quality; recognition of problems and action to be taken

- problems that can occur with the welding activities; materials and weld defects

- self inspection of completed work

- organisational quality systems (standards to be achieved; production records to be kept)

- personal approval tests and their applicability to your work

- the extent of your own authority and whom you should report to if you have problems that you cannot resolve

- reporting lines and procedures, line supervision and technical experts

Scope/range related to performance criteria

​

- Ensure the resistance spot welding equipment is fit for purpose, by carrying out all of the following checks:

  - equipment range is suitable for the operations to be performed

  - portable equipment power leads are undamaged and securely connected

  - electrodes are of the correct type, size and profile

  - all equipment mechanical and electrical systems operate correctly

  - supplies of components are adequate and suitably prepared

  - appropriate safety screens are available

- Set up, check, adjust and operate one of the following resistance spot welding machines:

  - portable spot welding machines

  - fixed simple spot welding machines

- Set up the equipment parameters in accordance with instructions and the welding procedure specification, to include setting all of the following:

  - electrode tip diameter/profile

  - welding current

  - welding and squeeze times

  - electrode pressure

  - water cooling flow rate (as applicable)

- Monitor the process operation and make adjustments to parameters, in order to produce welded components covering both of the following:

  - two different components

  - two different material thicknesses

- Produce welded components which meet all the following requirements:

  - achieve a weld quality as specified in the application standard

  - spot welds are correctly pitched out

  - welded components meet the required dimensional accuracy within specified tolerance

Behaviours

​​Additional Information

You will be able to apply the appropriate behaviours required in the workplace to meet the job profile and overall company objectives, such as:

- strong work ethic

- positive attitude

- team player

- dependability

- responsibility

- honesty

- integrity

- motivation

- commitment

Developed by

Enginuity

Version Number

3

Date Approved

30 Mar 2017

Indicative Review Date

31 Mar 2020

Validity

Current

Status

Original

Originating Organisation

Semta

Original URN

SEMFWE230

Relevant Occupations

Engineering, Engineering and Manufacturing Technologies, Metal Forming, Welding and Related Trades

Suite

Fabrication and welding suite 2

Keywords

engineering; welding; fabrication; joining; spot weld; spot welder; freestanding spot welder; parameters; monitor; weld condition