| SEMFWE334 Cutting materials using saws and abrasive discs |  |
| --- | --- |

Overview

This standard identifies the competencies you need to cut and shape materials using saws and abrasive discs in accordance with approved procedures. You will be required to select the appropriate equipment for the operations to be carried out and check that it is in a safe and usable condition. In carrying out the cutting and shaping operations you will be expected to use both saws and abrasive discs to cut and shape the materials to the required accuracy and specification.

Your responsibilities will require you to comply with organisational policy and procedures for the cutting activities undertaken and to report any problems with the equipment or the cutting activities that you cannot personally resolve, or are outside your permitted authority, to the relevant people. You will be expected to work with minimum supervision, taking personal responsibility for your own actions and the quality and accuracy of the work that you carry out.

Your underpinning knowledge will provide a good understanding of your work, and provide an informed approach to applying cutting procedures when using saws and abrasive discs. You will understand the suitability of the cutting processes, and their applications, and will know about the characteristics of the materials and the appropriate processes and techniques in adequate depth to provide a sound basis for carrying out the activities to the required specification. You will understand the safety precautions required when carrying out the cutting and shaping activities. You will be required to demonstrate safe working practices throughout, and will understand the responsibility you owe to yourself and others in the workplace.

Performance criteria

You must be able to:

- work safely at all times, complying with health and safety and other relevant regulations and guidelines

- confirm that the machine is set up and ready for the machining activities to be carried out

- manipulate the machine tool controls safely and correctly in line with operational procedures

- produce cut components to the required quality and within the specified dimensional accuracy

- carry out quality sampling checks at suitable intervals

- deal promptly and effectively with problems within your control and report those that cannot be solved

- shut down the equipment to a safe condition on conclusion of the machining activities

Knowledge and understanding

You need to know and understand:

- the specific safety precautions to be taken when working in a fabrication environment and when working with power operated saws and abrasive disc cutting machines (statutory regulations, risk assessment procedures and COSHH regulations)

- the personal protective clothing and equipment (PPE) that needs to be worn when carrying out the fabrication activities (such as leather gloves, eye/ear protection, safety helmets)

- safe working practices and procedures to be observed when working with the machines including emergency shutdown procedures

- the correct methods of moving or lifting heavy plate or rolled sections

- the hazards associated with fabrication work and cutting operations and how they can be minimised (such using dangerous or badly maintained tools and equipment; airborne particles; hot metal; burrs and sharp edges)

- how to obtain the necessary drawings, specifications and work instructions

- how to use and extract information from engineering drawings and related specifications (to include symbols and conventions to appropriate British, European or relevant International standards in relation to work undertaken)

- how to interpret first and third angle drawings, imperial and metric systems of measurement, workpiece reference points and system of tolerancing

- how to interpret marking out conventions (such as cutting lines, centre lines)

- the range of machine saws available (such as power hacksaws, circular saws and band saws)

- the abrasive cutting equipment available to include hand held portable machines and bench type radiac cutting machines

- the selection and fitting of abrasive cutting discs, cutting disc identification markings and how to identify the correct type of disc for the type of material being cut

- statutory regulations regarding the fitting and use of abrasive discs

- the material cutting characteristics and process considerations that need to be taken into account when cutting materials

- the use and care of tools and equipment such as checking that trailing leads, plugs and sockets are in a safe, tested and usable condition

- the use of safety screens to protect other uses from flying sparks whilst using abrasive cutting discs

- the importance of ensuring that the machine guards are correctly fitted and positioned before using the equipment

- how to set and adjust power saws for the various operations being performed

- the importance of using tools or equipment only for the purpose intended; the care that is required when using the tools or equipment; the proper way of preserving tools or equipment between operations

- the problems that can occur when cutting materials using saws or abrasive discs, and how these can be avoided

- inspection techniques that can be applied to check shape and dimensional accuracy is to specification and within acceptable limits

- the extent of your own responsibility and who you should report to if you have problems that you cannot resolve

- reporting lines and procedures, line supervision and technical experts

Scope/range related to performance criteria

​

- Ensure that the equipment is fit for purpose and used safely by carrying out

- all

- of the following:

  - selecting the appropriate equipment/machine for the operation being performed

  - checking the machine guards and safety devices are in position and function correctly

  - checking cutting discs/blades are in a serviceable condition (free from damage or chips; sharp)

  - isolating the equipment from its power supply whilst changing blades or discs

  - using the equipment safely and correctly and only for its intended purpose

- Use

- two

- of the following types of cutting equipment:

  - machine saw

  - hand held portable abrasive disc

  - band saw

  - radiac abrasive disc

- Carry out

- all

- of the following cutting and shaping activities:

  - straight sawing

  - abrasive disc cutting

  - contour shaping using saws

- Cut and shape components which contain

- all

- of the following features:

  - straight parallel cuts

  - curved contours

  - square cuts

  - angled/mitred cuts

- Cut and shape

- three

- of the following forms of material:

  - flat plate

  - solid bar (square, round, hexagonal)

  - rolled sections (angle, channel, RSJ )

  - pipe/tube

  - rail section

  - non-ferrous materials

  - other specific form

- Produce components that comply with

- all

- of the following quality and accuracy standards:

  - dimensional accuracy is within specification tolerances

  - cuts are square and clean and free from excessive burrs

  - angled cuts are within specification requirements

Behaviours

​Additional Information

You will be able to apply the appropriate behaviours required in the workplace to meet the job profile and overall company objectives, such as:

- strong work ethic

- positive attitude

- team player

- dependability

- responsibility

- honesty

- integrity

- motivation

- commitment

Developed by

Enginuity

Version Number

3

Date Approved

30 Mar 2017

Indicative Review Date

31 Mar 2020

Validity

Current

Status

Original

Originating Organisation

Semta

Original URN

SEMFWE3-34

Relevant Occupations

Engineering, Engineering and Manufacturing Technologies, Metal Forming, Welding and Related Trades

Suite

Fabrication and Welding Engineering Suite 3

Keywords

engineering; welding; fabrication; machining; cutting; sawing; abrasive disc; machine saw; band saw; radial abrasive disc