| SEMAE3320 Maintaining pneumatic systems on aircraft |  |
| --- | --- |

Overview

This standard identifies the competences you need to carry out maintenance activities on aircraft pneumatic systems, in accordance with the aircraft maintenance manual, approved change documentation (service bulletin) and airworthiness requirements. It covers both fixed wing and rotary winged aircraft, and covers the units and components which deliver large volumes of compressed air from a power source to connecting points for other systems, such as air conditioning, pressurization and de-icing. The maintenance activities will include the removal, fitting and testing of a range of pneumatic components. You will be expected to use the approved procedure for correctly isolating and de-pressurising the system before breaking into the system circuit. You will remove the required components and fit approved replacements, as appropriate. You will then need to pressurise the system, and test and adjust the completed system to meet the aircraft maintenance manual, change documentation (service bulletin) and airworthiness requirements.

Your responsibilities will require you to comply with the specific practices and procedures identified in the aircraft manual, change/service bulletin documentation and airworthiness requirements for the maintenance activities undertaken, and to report any problems with these activities that you cannot personally resolve, or that are outside your permitted authority, to the relevant people. You must ensure that all tools, equipment and materials used are correctly accounted for on completion of the activities, and that all necessary job/task documentation is completed thoroughly, accurately and legibly.  You will be expected to work with a minimum of supervision, taking personal responsibility for your own actions and for the quality and accuracy of the work that you carry out.

Your underpinning knowledge will provide a good understanding of your work, and will provide an informed approach to applying the appropriate maintenance techniques and procedures to aircraft pneumatic systems. You will understand the removal, fitting and testing methods and procedures, and their application, along with the pneumatic system maintenance requirements. You will know how the equipment functions, the common problems that can occur, the purpose of the individual components and associated defects, in adequate depth to provide a sound basis for carrying out the maintenance activities, correcting faults and for ensuring that the equipment is maintained to the required standard.

You will understand the safety precautions required when working on aircraft pneumatic systems, especially those for isolating and depressurising the equipment. You will be required to demonstrate safe working practices throughout, and will understand your responsibility for taking the necessary safeguards to protect yourself and others in the workplace.

Notes:

- This standard is designed to cover the practical experience requirements of the Airline Transport Association (ATA) Chapter 36 Pneumatic.

- To display competence in this standard, it is necessary to both remove and fit aircraft pneumatic system components. You must remove components; however, you may fit a replacement component where the original was previously removed by another person. You should also be aware of how to leave a system in a safe condition if maintenance tasks cannot be completed. This covers both the physical systems and the job documentation.

Performance criteria

You must be able to:

- work safely at all times, complying with health and safety and other relevant regulations, directives and guidelines

- follow the relevant maintenance schedules to carry out the required work

- carry out the maintenance activities within the limits of your personal authority

- carry out the maintenance activities, and replace components in the specified sequence and in an agreed timescale

- report any instances where the maintenance activities cannot be fully met or where there are identified defects outside the planned schedule

- complete relevant documentation in accordance with organisational requirements

- dispose of waste materials in accordance with safe working practices and approved procedures

- leave the aircraft and system in a safe and appropriate condition, free from foreign object debris on completion of the activities

​

Knowledge and understanding

You need to know and understand:

- the specific safety practices and procedures that you need to observe when working on aircraft pneumatic systems (including any specific legislation, regulations/codes of practice for the activities, equipment or materials)

- the need to check that cabin/cockpit switches, selectors and circuit breakers are in the correct position before applying any form of external power (such as electrical, hydraulic, air or vacuum)

- the importance of maintenance on aircraft pneumatic systems, and impact upon Extended Range Twin-Engine Operations Procedures (ETOPS) systems, Electrical Wiring Interconnect Systems (EWIS), legislation and local procedures

- the requirements and importance of understanding and applying human factors as defined by the regulatory requirements and the potential impact if these are not adhered to

- the hazards associated with carrying out maintenance activities on aircraft pneumatic systems, and with the tools and equipment used (such as the safe release of pressurised systems, traps from moving parts, misuse of tools), and how to minimise them and reduce any risk

- the protective equipment that you need to use for both personal protection (PPE) and protection of the aircraft

- what constitutes a hazardous voltage and how to recognise victims of electric shock

- how to reduce the risks of a phase to earth shock (such as insulated tools, rubber matting and isolating transformers)

- the importance of aircraft husbandry and of ensuring that, throughout the maintenance activity, the aircraft and work area are maintained free from foreign objects, and the implications of FOD to the safety of the aircraft

- how to extract and use information from aircraft manuals, log books, flight logs, charts, circuit and physical layouts, specifications, symbols used in aircraft pneumatic systems, and other documents in the maintenance activities

- how to carry out currency/issue checks on the specifications you are working with

- terminology used in aircraft pneumatic systems, and the use of system diagrams and associated symbols

- the various types of pipe and component that make up the aircraft pneumatic system (such as rigid pipes; air hoses; pipe connectors; pipe sealing and supporting devices; valves used for pressure relief, flow and directional control; pumps; pressure intensifiers, mechanical and electrical control devices)

- why electrical bonding is critical, and why it must be both mechanically and electrically secure

- the principles of operation of the pneumatic system being worked on (such as system layout, sources of air pressure (such as engine, compressor, ground supply); the use of air reservoirs/tanks; pressure control and distribution; pressure indication and warning)

- the techniques used to remove components from aircraft pneumatic systems without damage to the components or surrounding structure (such as release of pressures/force, removal of components and the need to protect the circuit integrity by ensuring that any exposed components or pipe ends are correctly covered/protected)

- the various mechanical fasteners that will need to be removed and replaced, and their methods of removal and replacement (such as threaded fasteners, special securing devices)

- the various types of electrical connector that are used, methods of unlocking, orientation indicators and locating and locking-in of the connections

- recognition of contaminants and the problems they can create; the effects and likely symptoms of contamination in the pneumatic system

- the need to label and store correctly components that require repair or overhaul, and to check that replacement components have the correct part/identification markings and accompanying release documentation

- how to fit components into the circuit (such as the use of gaskets/seals and jointing/sealing compounds; ensuring the correct tightness of pipe fittings and pump connections; eliminating stress on pipework/connections; carrying out visual checks of all components; checking the security of joints and that the system is safe to re-pressurise)

- the tools and equipment used in the maintenance activities, and their calibration/care and control procedures

- why tool/equipment control is critical and what to do if a tool or piece of equipment is unaccounted for on completion of the activities

- how to make adjustments to components/assemblies to ensure that they function correctly (such as pressure settings and their effect on the system, travel and working clearance)

- why securing devices need to be tightened to the correct torque, locked and labelled, and the methods to be used

- how to carry out routine checks and servicing of the aircraft pneumatic system (including checking for leaks, checking and changing air filters, charging desiccators, checking and adjusting pressure regulator)

- the types of test to be carried out on the aircraft pneumatic system and the test equipment to be used

- the methods and procedures to be used to carry out the various tests on the pneumatic system

- the need to apply test pressures in incremental stages, and to check all readings and pressures at each stage

- how to record the results of each individual test, and the documentation that must be used

- how to analyse the test results, and make valid decisions about the acceptability of the aircraft pneumatic system

- the procedures to be followed if the equipment or system fails to meet the test specification

- the recording documentation to be completed for the activities undertaken and, where appropriate, the importance of marking and identifying specific pieces of work in relation to the documentation

- the procedure for the safe disposal of waste materials and scrap components

- the extent of your own authority and to whom you should report if you have problems that you cannot resolve

Scope/range related to performance criteria

​

- Carry out all of the following during the maintenance of the aircraft pneumatic system:

  - ensure that appropriate authorisation to work on the aircraft is obtained, and observe all relevant isolation and safety procedures

  - obtain and use the correct documentation (such as job instructions, technical instructions, aircraft manuals and maintenance documentation)

  - obtain the correct tools and equipment for the activity, and check that they are in a safe, tested and usable condition and within current calibration dates

  - adhere to procedures or systems in place for risk assessment, COSHH, personal protective equipment and other relevant safety regulations and procedures to realise a safe system of work

  - ensure the safe isolation and depressurisation of the pneumatic equipment before breaking into the system

  - ensure that the relevant safety devices and mechanical/physical locks are in place (where appropriate)

  - use approved removal, fitting and testing techniques and procedures at all times

  - return tools and equipment to the correct storage location on completion of the activities

  - ensure that work carried out is correctly documented and recorded

  - ensure that any outstanding tests are correctly documented

- Carry out maintenance on two of the following parts of the aircraft pneumatic system:

  - pneumatic supply

  - indicating

  - distribution

  - gauging

- Remove and fit four different aircraft pneumatic system components (at least one must be from group A):

Group A

‍1.  pump                
‍2.  air reservoirs/tanks    
‍3.  manifold
‍4.  compressors        
‍5.  accumulator  
‍6.  desiccators
‍7.  standby pump         
‍8.  oil and water trap 
‍9.  heat exchangers

Group B

10. control valves

11. pipes and hoses

12. wiring/switches/plugs

13. check valve

14. ducting

15. pressure regulating valve

16. ground connector

17. air filters

18. warning devices (temperature, pressure)

19. pressure relief valve

20. sensors/transmitters

21. actuators

22. gauges

23. control mechanisms

24. other specific components

- Carry out fifteen of the following maintenance activities:

  - removing access panels and covers to expose components to be removed

  - carrying out fault diagnosis and system checks

  - preparing the system for maintenance (such as isolating, de-pressurising)

  - disconnecting electrical connections

  - refitting components in the correct position, orientation and alignment

  - removal of bonding

  - disconnecting/removing hoses and pipes

  - setting and adjusting replaced components (such as travel, working clearance)

  - removing securing devices and mechanical fasteners

  - supporting equipment to be removed

  - making mechanical connections

  - dismantling equipment to an appropriate level

  - making electrical connections

  - covering (protecting) exposed components, wires, pipework or vents

  - carrying out bonding

  - torque loading as required

  - checking components for serviceability

  - re-pressurising the system

  - replacing damaged/defective components

  - carrying out system functional checks

  - replacing single use items such as seals, filters, gaskets

  - ensuring that replacement components have the correct part numbers

  - fitting blanks, labelling (and storing in the correct location) components that require repair or overhaul

  - applying bolt locking methods (such as split pins, wire locking, lock nuts)

- Service/check the aircraft pneumatic system, to include carrying out four of the following:

  - checking the system for leaks

  - replacing filters

  - recharging desiccators

  - checking and adjusting pressure regulator

  - checking the indicating systems (such as pressure, temperature warning)

- Carry out three of the following tests on the aircraft pneumatic system:

  - leak test

  - reduced system test

  - pressure test

  - movement tests (such as range, timing, sequencing)

  - system charging

  - built in test equipment (BITE) test

  - air line vapour tests

  - 'special-to-type' tests

Using one of the following:

9. aircraft power source/pumps

10. ground test rig

- Complete the relevant paperwork, to include one from the following and pass it to the appropriate people:

  - job cards/work sheets

  - computer records

  - aircraft technical log

  - aircraft cabin log

  - aircraft log book

- Carry out maintenance on aircraft pneumatic system components in compliance with one of the following:

  - Civil Aviation Authority (CAA)/European Aviation Safety Agency (EASA)

  - Extended Range Twin-Engine Operations Procedures (ETOPS) (where appropriate)

  - Ministry of Defence (MoD)

  - Military Aviation Authority (MAA)

  - Aerospace Quality Management Standards (AS)Federal Aviation Authority (FAA)

  - aircraft maintenance manual/approved change documentation (service bulletin)

  - manufacturers standards and procedures

Behaviours

​

You will be able to apply the appropriate behaviours required in the workplace to meet the job profile and overall company objectives, such as:

- strong work ethic

- positive attitude

- team player

- dependability

- responsibility

- honesty

- integrity

- motivation

- commitment

Developed by

Enginuity

Version Number

3

Date Approved

30 Mar 2021

Indicative Review Date

01 Mar 2024

Validity

Current

Status

Original

Originating Organisation

Enginuity

Original URN

SEMAE3320

Relevant Occupations

Engineer, Engineering, Engineering and Manufacturing Technologies, Engineering Technicians

Suite

Aeronautical Engineering Suite 3

Keywords

engineering; aeronautical; aircraft pneumatic systems; compressed air; air conditioning; pressurization; de-icing; de-pressurising